16.412 Tutorial Outline

- Nature of the problem:

  - harsh/boring jobs  robots instead of humans

  - need for heterogeneous robot teams, benefits over monolithic robots (cheaper, more flexible, etc)

  - real world  dynamic environments, changing robot abilities

- Design goals

  - Robots should be able to respond to changing environment, changing abilities of self & others

  - Problems must be solved within time constraints  no “swarm strategies”

- Strategy

  - Utilize global rather than local rules

  - All robots communicate with all others

  - Behavioral sets are given motivation levels based on environment, perceived abilities of self and others

  - Most able robot is given responsibility for task

  - Ability may change, this change is determined based on perceived success in affecting environment

- Assumptions

  - Robots can detect the effect of their own actions

  - Robots can detect actions of other team members

  - Robots don’t lie to each other

  - Communications may not be available

  - Sensors aren’t perfect

  - Any robot can fail

  - Failure cannot necessarily be communicated

  - There is no centralized store of knowledge

- ALLIANCE Architecture

  - Fully distributed within team of fault-tolerant, heterogeneous robots

  - Robot action selection based on:

    - Mission requirements

    - Actions of other robots

    - Current environment

    - Own internal state

  - Behavior-based approach to achieve robustness at individual robot level

    - Several task-achieving behaviors active simultaneously

    - Each receives sensory input, controls some part of actuator output

    - Lower level behaviors (obstacle avoidance, etc) always active

    - Lower level behaviors may be suppressed by task-achieving behaviors

  - Extensions – how does robot select among competing actions?

    - Several behavior sets exist, always either active or hibernating

    - Select appropriate behavior set through motivational behaviors

- Motivational Behaviors

  - Each motivational behavior receives several inputs

    - Sensory feedback

    - Inter-robot communication

    - Inhibitory feedback from other robots

    - Internal motivations – robot impatience and acquiescence

  - Output is activation level of corresponding behavior set (active or not active)

  - Works by increasing motivation level for a certain task, rate proportional to current accomplishment level of that task by team

  - Impatience:  sensory feedback overrides feedback from other robots – corrects faults in other robots

  - Acquiescence:  if sensory feedback indicates robot is not successful at task, motivation level decreases until another robot takes over task

  - Motivations fluidly adapt to changes in environment

- Formal Model of ALLIANCE

  - Set R of robots, set T of tasks in mission, set A of behavior sets for robot in R

  - Set of n functions which return task in T that robot in R is working on when behavior in set A is activated

  - Sensory_feedback(t) returns 1 or 0, provides motivational behavior with data needed to decided whether to activate corresponding behavior set

  - Comm_received(I,k,j,t1,t2) returns 1 or 0 depending on presence of communication messages within time span

  - Activity_suppression(t) returns 1 or 0, depending on whether or not other behavior sets are already active

  - Impatience(t) returns a fast or slow rate based on feedback regarding the accomplishment of a task

  - Impatience_reset(t) returns a 1 or 0, resetting the impatience level when either the current robot or another robot has just begun to perform a certain task

  - Acquiescence(t) returns a 1 or 0, indicating whether or not a robot has decided to give up its current task

  - The initial motivation level m(0) of any task is equal to 0

  - The motivation level m(t) = [m(t-1)+impatience(t)]*(product of sensory feedback, activity suppression, impatience reset, and acquiescence)

  - Key parameters affecting action selection:

    - Rate of impatience (fast)

    - Rate of impatience (slow)

    - Time that robot maintains activity before acquiescence

- Example – hazardous waste cleanup

  - Two-robot team

  - One robot has been partially disabled, cannot follow walls efficiently  allows other robot to do mapping

  - Other robot turns out to be more disabled  partially disabled robot takes over mapping responsibilities

  - Both robots do cleanup after mapping is complete

  - When one robot reports results, other robot is no longer motivated to do so