The dtran program monitors the terminals which drive the nets attached

to the dtran (we will call these terminals "drivers").  It has a data

structure which holds the number of drivers on each side, the state of

those drivers and the types of nets (WOR, WAND, WIRE) which drive the

dtran.

When a driver changes value, the consumer routine updates the data

structure and schedules the misctf routine.  The misctf routine

evaluates the data structure and puts the new values into the

registers in the dtran module.

THE DATA STRUCTURE

The dtran data structure contains 4 integers and two integer arrays.

The integers contain the number of drivers and the arrays contain the

values of the drivers.

For example:

|\

| \

| <1>-----+

| /       |       dtran                 /|

|/|       |      +-------+             / |

|-wor--|a     b|---wire----<Z> |

|      +-------+	            \ |

|\	       |                            |\|

| \       |

| <0>-----+

| /

|/|

The data structure will look like this for the above picture:

typedef struct {

int

number_of_a_drivers, = 2

number_of_b_drivers, = 1

*a_values,           = [1, 0]  /* we will talk about these value */

*b_values,           = [Z]     /* representations later */

a_nettype,           = WOR

b_nettype;           = WIRE

} dtran_inst_info;

We see that there are 2 drivers on the a port and that the a netttype

is WOR.  There is 1 driver on the b port and that nettype is WIRE.

The values of the drivers are stored in two arrays.  a_values contains

the values for the a drivers and b_values contains the values for the

b driver.

The checktf routine sets up a vcl monitor on all these drivers.  It

passes the consumer routines a pointer to the place in the a_values

or b_values array which corresponds to that driver.

When a driver changes value, the vcl consumer routine modifies the

value array and places the misctf routine on the event queue.  The

misctf routine evaluates the arrays and decides which values should be

driven onto the a and b ports of the dtran.  Then it stores that value

in the a_reg or b_reg in the dtran module.

EVALUATING THE VALUE ARRAYS

We want the consumer routines and the misctf routines to be as

efficient as possible, so we need a quick way of evaluating the

drivers and determining the value that should be driven onto the

opposite net.

We accomplish this efficiency with a system of storing values that

lets us evaluate a value array by bitwise-ORing all of the array

elements together.  The value that results is the new value of the

net.

The value representations are the basis of this system.  These

representations are based upon the value being driven onto the net,

and the nettype.  Here are the representations, the numbers are in

binary format:

Wire		WOR		WAND

0 	010		100		111

1	100		111		100

X	110		101		101

Z	000		000		000

Let's look at our example again:

|\

| \

| <1>-----+

| /       |       dtran                 /|

|/|       |      +-------+             / |

|-wor--|a     b|-----------<z> |

|      +-------+	            \ |

|\	       |                            |\|

| \       |

| <0>-----+

| /

|/|

This time I've stored the values in a_values and b_values with the

proper format (in binary):

typedef struct {

int

number_of_a_drivers, = 2

number_of_b_drivers, = 1

*a_values,           = [111,100]

*b_values,           = [000]

a_nettype,           = WOR

b_nettype;           = WIRE

} dtran_inst_info;

The b_value evaluates with no problem, it has only one element.  Let's

look at the a_value:

[111, 100]

If we bitwise-OR these values we get:

111 | 100  = 111   or 1

This is the way a WOR net works.  Let's try some other examples.

a WOR with 3 drivers, two are 0 and one is X:

100 | 100 | 101 = 101  -> X (101 = X on a WOR net, see table)

a WAND with 3 drivers, two are 0 and one is X:

111 | 111 | 101 = 111  -> 0 (111 = 0 on a WAND net, see table)

a WIRE with three drivers, a 1, a 0, and a Z:

100  | 010 | 000 = 110  -> X  (110 = X on a WIRE net, see table)

The misctf routine knows what kind of net it is evaluating, so it

knows how to interpret the results of the bitwise-or.  This is why the

a_nettype and b_nettype are part of the data structure.  The consumer

routine also receives the nettype so it knows what kind of values to

put in the value arrays.

Assume that the <1> in the above example changes to a 0.  Let's follow

the steps of the process:

1.	The conumer routine for the upper terminal above gets called.

It takes the new value of the terminal (0) and stores it into

the a_value array.

The situation looks like this:

|\

| \This driver just changed.

| <0>-----+

| /       |       dtran                 /|

|/|       |      +-------+             / |

|-wor--|a     b|-----------<z> |

|      +-------+	            \ |

|\	       |                            |\|

| \       |

| <0>-----+

| /

|/|

The data structure will look like this for the above picture:

typedef struct {

int

number_of_a_drivers, = 2

number_of_b_drivers, = 1

*a_values,           = [100, 100]

^ The consumer routine has a pointer to

this memory location.  It changed the

value to 100 because it saw the new

driver value was 0 and it knows that

the net is of type WOR.

*b_values,           = [z]     /* representations later */

a_nettype,           = WOR

b_nettype;           = WIRE

} dtran_inst_info;

Now the consumer routine schedules the misctf routine by

calling tf_synchronize.

2.	The misctf routine evaluates the a_value array.  It does a

bitwise-or of the a_value array and gets 100 as the result.

Since it knows that nettype a is a WOR, it stores a 0 in

the b_reg in the dtran module.  The a_value goes into the

b_reg because we want to transfer the value to the other side

of the dtran.

Now the misctf routine evaluates the b_value array.  Since

there is only one value this quickly evaluates to 000.

The misctf routine knows that 000 is Z on a WIRE nettype, so

it stores a Z in the a_reg.

3.	The rest is up to Verilog-XL.  The dtran drives a Z onto the

a-side net.  This Z loses contention against the two 0 drivers

so the a-side net settles as a 0.  The dtran drives a 0 onto

the b-side net.  This 0 overpowers the Z being driven onto the

net, so the b-side net settles as a 0.  Both nets agree, and

we have achieved tran functionality.

STORING THE DATA STRUCTURES

We allocate the memory for the data structure at run time in the

checktf routine.  We have to do this because the a_value and b_value

arrays can be different sizes.

Each instantiation of the dtran gets its own data structure.  We store

a pointer to that data structure in the workspace for the

instantiation.

The consumer routine data structure is also allocated at run time.  We

store the pointer to this data structure in the user_data argument of

the acc_vcl_add call.