The frame grabber can capture and continually redisplay a frame of

512x512 bytes, with frame timing that is essentially that of VGA.

The beginning of a new frame is given by the VCAM pulse (derived from

the Sync message stream); during the entire frame, the grabber will be

either in sample mode or in hold mode, depending on whether DispAct

was low or high at VCAM time.

In sample mode, the grabber forwards to the videoDAC the video data

coming from CAM via the Disp0--Disp7 lines, while saving a copy of

this data in a videoDRAM frame buffer.  In hold mode, the grabber

ignores CAM (which is presumably not sending video data) and replays

to the videoDAC the frame buffer's contents.

The videoDAC's eight data inputs are given by the FrameData bus, which

in sample mode is driven for the entire frame by Disp0--Disp7 via a

bus buffer, and in hold mode is driven (during the active portions of

a frame) directly by the videoDRAM.

The frame buffer consists of two nybble-wide 256K VideoRAMs, ganged to

give byte width. For each active line of a frame,

---In sample mode, video data are shifted into the videoDRAM's

serial register, after which they are written broadside to the

appropriate DRAM row.

---In hold mode, a DRAM row is read broadside into the serial

register at the beginning of the line, after which the register is

shifted out.

---In either mode, two refresh cycles are performed at the end

of the line.

The 9 bits of row address (RAS time) are provided by the vertical

counter, VCOUNT. On each row, the initial column for a serial transfer

is given by 9 bits of column address (CAS time). As long as serial

input and serial output for a given row start at the same column, the

inital column address is irrelevant; for simplicity, the CAS address

is made equal to the RAS address.

The videoDRAM is by default (in particular, during horizontal and

vertical retraces) in serial input mode. It goes into serial output

mode (where it drives the FrameData bus) only during the active

portion of an active line of an hold-mode frame. In this way,

contention for the FrameData bus is avoided by a wide margin.

The RX PAL decodes the Sync message stream and generates (in addition

to sync signals for the monitor) an HTAB pulse at the beginning of the

active portion of each line; this pulse is accompanied by a VTAB pulse

on the first active line of each frame. The actual video data of each

line only start a few clock cycles after HTAB (the delay is determined

by the DELAY PAL), and are immediately preceded by the HTRIG pulse.

HTRIG triggers off a ``horizontal run'' window of 512 cycles, HRUN,

which exactly encompasses the video data.

The sequence of operations on each line is the following (the first

column refers to the state of the VRAM PAL state machine)

WAIT	wait for HTAB

RT	on HTAB, initialize serial data transfer

WAIT	wait for HRUN

RUN	perform serial transfer during HRUN

WT	after end of HRUN, finalize serial data transfer

RF	perform two refresh cycles

...	go back to WAIT