#cell2 * chip doc * 3 any 0 v8r2.2

# "21-Feb-92 GMT" "15:44:47 GMT" "21-Feb-92 GMT" "15:44:47 GMT" td * .

# General Information:

system_name                SPARC UNIX

user_Id                    td

home_directory             /im/td

mainsail_version           11.27

vlsi_tools_version         v8r2.2

tool                       Gate Compiler

technology                 cmn12

last_operation             Eval FloorPlan

design                     chip_f

# Nets And Cells:

nets                       7268

arbitrary_blocks           0

cell_areas                 5

gatearray_cells            6932

pads                       66

pins_per_net               3.59

nets_with_weight           190

nets_with_maxcap           0

net_with_max_pins          S0 71

clock_net                  CLK

# Total Area:

base                       vgt300077

device_number

overall_occupancy          92.767%

package                    P2367506

chip_size                  14401.00 X 14401.00 lambda  (340.18 X 340.18 mils)

core_size                  12320.00 X 12208.00 lambda  (291.02 X 288.38 mils)

transistors                109842

core_density               .5298E-03 trans/sq-lam (.2374 gates/sq-mil)

# Routing Factor:

block_routing_factor       .43

core_routing_factor        1.88

padring_routing_factor     1.39

# Post Route:

vias                       0

contacts                   0

metal1_wire_length         5911567 lambda

metal2_wire_length         7180546 lambda

poly_wire_length           0 lambda

unroutes_in_cell_routing   0

unroutes_in_block_routing  0

# Cell Areas:

Area u1054_1

size                       4041 X 12210 lambda

routing_density            .81

occupancy                  96.817 %

routing_factor             1.03

transistors                38025

density                    .7707E-03 trans/sq-lam (.3453 gates/sq-mil)

gatearray_cells            2362

end_area

Area u1054_2

size                       3740 X 12210 lambda

routing_density            .58

occupancy                  97.140 %

routing_factor             1.03

transistors                33836

density                    .7410E-03 trans/sq-lam (.3320 gates/sq-mil)

gatearray_cells            2426

end_area

Area u10

size                       4223 X 5490 lambda

routing_density            .75

occupancy                  95.746 %

routing_factor             1.00

transistors                17642

density                    .7609E-03 trans/sq-lam (.3409 gates/sq-mil)

gatearray_cells            845

end_area

Area u8

size                       4146 X 3530 lambda

routing_density            .88

occupancy                  92.788 %

routing_factor             .98

transistors                11277

density                    .7705E-03 trans/sq-lam (.3452 gates/sq-mil)

gatearray_cells            596

end_area

Area u9

size                       4195 X 2970 lambda

routing_density            .91

occupancy                  93.231 %

routing_factor             .98

transistors                9062

density                    .7273E-03 trans/sq-lam (.3259 gates/sq-mil)

gatearray_cells            703

end_area

# Run Time:

run_time Evaluate Floor Plan 111.45 sec on 20 February 1992 at 22:46:47