#cell2 * chip doc * 3 any 0 v8r2.2

# "23-Feb-92 GMT" "23:37:58 GMT" "23-Feb-92 GMT" "23:37:58 GMT" td * .

# General Information:

system_name                SPARC UNIX

user_Id                    td

home_directory             /im/td

mainsail_version           11.27

vlsi_tools_version         v8r2.2

tool                       Gate Compiler

technology                 cmn12

last_operation             Eval FloorPlan

design                     chip_f

# Nets And Cells:

nets                       7268

arbitrary_blocks           0

cell_areas                 5

gatearray_cells            6932

pads                       66

pins_per_net               3.59

nets_with_weight           190

nets_with_maxcap           0

net_with_max_pins          S0 71

clock_net                  CLK

# Total Area:

base                       vgt300077

device_number

overall_occupancy          93.212%

package                    P2367506

chip_size                  14401.00 X 14401.00 lambda  (340.18 X 340.18 mils)

core_size                  12320.00 X 12208.00 lambda  (291.02 X 288.38 mils)

transistors                109842

core_density               .5298E-03 trans/sq-lam (.2374 gates/sq-mil)

# Routing Factor:

block_routing_factor       .42

core_routing_factor        1.88

padring_routing_factor     1.39

# Post Route:

vias                       0

contacts                   0

metal1_wire_length         5875290 lambda

metal2_wire_length         5105628 lambda

poly_wire_length           0 lambda

unroutes_in_cell_routing   0

unroutes_in_block_routing  0

# Cell Areas:

Area chip_ga1

size                       5441 X 7730 lambda

routing_density            .64

occupancy                  96.190 %

routing_factor             1.04

transistors                31088

density                    .7392E-03 trans/sq-lam (.3312 gates/sq-mil)

gatearray_cells            2111

end_area

Area chip_ga

size                       6582 X 7730 lambda

routing_density            .98

occupancy                  96.201 %

routing_factor             1.04

transistors                38953

density                    .7656E-03 trans/sq-lam (.3430 gates/sq-mil)

gatearray_cells            2598

end_area

Area chip_ga2

size                       3488 X 4370 lambda

routing_density            .63

occupancy                  94.829 %

routing_factor             1.01

transistors                11534

density                    .7567E-03 trans/sq-lam (.3390 gates/sq-mil)

gatearray_cells            610

end_area

Area chip_ga3

size                       3152 X 4370 lambda

routing_density            .65

occupancy                  94.806 %

routing_factor             1.02

transistors                9876

density                    .7170E-03 trans/sq-lam (.3212 gates/sq-mil)

gatearray_cells            745

end_area

Area chip_ga4

size                       5539 X 4370 lambda

routing_density            .84

occupancy                  96.168 %

routing_factor             1.01

transistors                18391

density                    .7598E-03 trans/sq-lam (.3404 gates/sq-mil)

gatearray_cells            868

end_area

# Run Time:

run_time Evaluate Floor Plan 41.91 sec on 22 February 1992 at 16:28:27