SBus PROTOCOL MONITOR and

DRIVER ROUTINES

User's Guide

Todd Wayne

pokey@spcmnspf

Document Revision 0.1

Copyright 1989 Sun Microsystems, Inc.  All Rights Reserved.

TABLE of CONTENTS

1.		Features

1.1		    Partition Overview

1.2		    Monitor Overview

1.3		    Driver Overview

2.		Block Diagram(s)		(Not Yet available...)

2.1		    ASIC Test Block Diagram

2.2		    System Test Block Diagram

3.		Pin Description

4.		Electrical Specification

5.		Pin Timing

6.		Functional Descriptions of Sub-Units

6.1		    Monitor Module

6.1.1		        State Diagrams		(Not Yet Available in ASCII)

6.1.2			Error Messages

6.2		    Driver Routines

6.2.1			User Definable Variables and Memory

6.2.2			Driver Tasks

7.		Timing Diagrams

8.		Revision History

SBus Monitor/Driver FUNCTIONAL SPECIFICATION

This document provides a detailed description of the SBus Protocol

Monitor and Driver Routines designed for testing the SBus Interface

ASIC's.  It assumes that the User is familiar with SBus concepts and pin

names.  Descriptions are at the state diagram, and functional level.

Another relevant document is the SBus Specification (authored by Ed

Frank).

1.  Features ("What it does")

+ 16 Driver Routines to simulate various aspects of the system needed to

test an ASIC or SBus Card.

+ A Protocol Monitor that stops (verilog) simulation with an error

message indicating the protocol violation.  Peculiarities of the

SparcStation 1 SBus are NOT accounted for - currently the Protocol

Monitor uses the Revision A specification as it's sole reference.

+ Written in purely behavioral Verilog code, for use with modified

Verilog version 1.4a which has been compiled with a $drivenp(pin);

routine to facilitate certain error checks.

+ Simple to instantiate and use.

+ Tasks are available for displaying revision number and for defining

log_file (output to file is in Monitor only).

+ Basic Memory subsystem complements Driver Functionality.

1.1.  Partition Overview

The Monitor and Driver routines operate on all devices associated with

the SBus.  The Monitor itself is not a device per say - it's inputs are

all of the wires that constitute the physical SBus, and it's output is

text only.  The Driver routines generate stimulus that simulates

whatever part of the system is needed to test a particular ASIC.  The

Monitor is for use during all design phases (with single or multiple

ASIC's), while the Drivers are intended for use with one ASIC at a time.

1.2.  Monitor Overview

All of the SBus wires are inputs to the SBus Monitor.  The type of data

transaction is determined based on the sampled activity, and when a bus

cycle is not completed according to the protocol specified in the

Revision A SBus Specification, simulation is stopped, and an error

message is written.

Error checking includes:

+ Accurate count of Dynamic Bus Sizing Followon Cycles (requiring

consistent acknowledge and port sizes).

+ Proper address incrementing during Dynamic Sizing.

+ Rerun only valid as first ack during Burst Transfers.

+ Rerun information preserved and tested for accuracy for all slaves on

the SBus (note: only 8 slaves supported).

+ Valid Bus Request line deassertion.

+ Appropriate drive strength and value on Acknowledge lines (and other

lines at various meaningful times).

1.3.  Driver Overview

SBus Driver Routines come in 3 flavors, since there are basically 3

perspectives from which an SBus transaction can be seen: System as

Master, Alternate Bus Master, and Slave.  When an SBus Controller chip,

for example, is under test, it will require routines to act as Slaves

and Alternate Masters, while an SBus card interface chip will require

routines to act as the System Master or Slaves.  All of the driver

routines are accessed by instantiating a single module into the top

level of hierarchy for the simulation, though not all of them will be

called by every chip.

The routines are Verilog tasks designed to be called as part of a test

pattern list.  Since they are normal verilog tasks, they can also be

used from the Interactive Verilog prompt, and they should not be

re-enabled (re-executed) before the previous attempt has finished

(Reference: Verilog XL Ref. Manual, Dec.88, Section 9.1.2, pp.137).

A user-definable address range (and slot number) will act as basic

memory.  Memory Size (and slot number) are controlled by Verilog

Parameters (fixed at compile time).  The two additional tasks `dump' and

`fill' operate on this memory.

The following list includes routine names and the device that

they are intended to simulate (memory related routines are explained

later):

Task:					Device Simulated:

-------------------------------------------------------------------------

ack					Slave (being written to)

ack_data				Slave (being read from)

read					System as Master (MSI)

burst_read				System as Master

write					System as Master

burst_write				System as Master

dma_master				System selecting Alternate Master

dma_slave_read				Alternate Master + Host Controller

dma_slave_burst_read			Alternate Master + Host Controller

dma_slave_write				Alternate Master + Host Controller

dma_slave_burst_write			Alternate Master + Host Controller

dma_alt_read				Alternate Master

dma_alt_burst_read			Alternate Master

dma_alt_write				Alternate Master

dma_alt_burst_write			Alternate Master

An SBus Slave will use the dma_slave_X and (burst_)read/write routines;

An alternate DMA Master will use the dma_master routine to give itself

the bus, and the ack(_data) routine to simulate the slave being

accessed; The System Master will use dma_alt_X routines to simulate

alternate masters, and the ack(_data) routine to simulate the slave's

response (or respond itself if appropriate).

2.   Configuration Diagrams ("How to connect the pieces")

These diagrams are not yet available in ASCII format.

2.1.   ASIC Test Block Diagram

2.2.   System Test Block Diagram

3.  Pin Description ("What to connect to the pieces")

The signals entering and leaving each behavioral cell are described

individually.

For those chips not required to test parity, 1'b0 can be used in the

PARITY port lists of the Monitor and Drivers.  I recommend that

registers (initialized in the top_level.v code) should be fed into

theses ports so that tests can easily interactively be toggled if parity

is necessary only at certian times.

3.1.  SBUSM.v Signals

SBus Monitor signals are identical to those of the SBus Specification

with several minor additions.  There are four extra pins for each of the

following signals: SB_BR_, SB_BG_, and SB_SEL_.  These extra pins are

intended for the 'geographical addressing' of potential SBus Masters (up

to 8, as per the RevA spec.)

The only other additional pin is PARITY.  It is an input designed to

trigger a parity checking routine in the Monitor code.  If the pin is

set to 1, parity checking will be enabled, otherwise the parity test

will not be in effect.  The parity test is described in the SBus Spec.

as XOR of SB_EXT with the reduction XOR of SB_DATA.

All Monitor pins are considered strictly as Inputs.

Instantiation of the module should place the pins in the following order:

[MODULENAME localname (pinlist);]

SBUSM sbusm (sb_clk, sb_as_, sb_pa(28), sb_d(32), sb_siz(3), sb_rd,

sb_ack_(3), sb_merr_, sb_br_(8), sb_bg_(8), sb_sel_(8), sb_irq_(7),

sb_reset_, sb_ext, parity_test_enable);

3.2.  SBUSD.v Signals

The Driver Routines pinlist is identical to that of the Monitor's, but

the pins are dealt with differently inside the code.  The Driver

routines consider all pins as Tri-State I/O except SB_RESET_ (which is

an Input that disables any active task that was asserted) and PARITY.

The PARITY pin will cause the SB_EXT pin to be driven with the correct

parity value if it is set to 1, and will cause SB_EXT to remain floating

(ie. never driven) if set to 0.  Naturally, this only has effect on

SB_EXT when the driver routines themselves are driving the SB_D lines,

and allows for forced parity error testing.

Instantiation of the module should place the pins in the following order:

SBUSD sbusd (sb_clk, sb_as_, sb_pa(28), sb_d(32), sb_siz(3), sb_rd,

sb_ack_(3), sb_merr_, sb_br_(8), sb_bg_(8), sb_sel_(8), sb_irq_(7),

sb_reset_, sb_ext, parity_generation_anable);

Bus-widths are (indicated) and must be maintained to assure correct

operation.  Other significant variables and tasks are described in

Chapter 6.

4.  Electrical Specifications

There are no true electrical specifications to the SBus Verilog code.

It should be noted that drive strength information is determined and

tested by the `$drivenp' Verilog PLI extension written by Raju Joshi.

Power Supply and Strongly Driven SBus lines will be considered "driven"

while all resistively pulled and high impedance lines will be considered

"undriven".  This is necessary for testing SBus Acknowledge and other

control signal lines.

As per the SBus Specification, several signal lines will have passive

pullups in the actual system.  These lines (including: SB_ACK_, SB_D,

SB_LATEERROR_, SB_BG_, and SB_BR_) should be declared as "tri1" in the

topmost level of Verilog code in simulations to insure proper function

of the $drivenp routine.

Please refer to the SBus Specification (by Ed Frank) for any other data

concerning SBus Electrical Specifications.

5.  Pin Timing

The SBus Monitor does not look for specific pin timing - it is driven

completely by the `rising edge' of SB_CLK (that is the only time most

signals are sampled - Rising {deactivating} Bus Requests are sample

asynchronously).  SBus Driver routines are designed to avoid timing

issues by applying and removing drive to pins based on the `falling

edge' of SB_CLK.  When it is necessary to keep track of clock ticks,

rising edges trigger those counters.

No Setup, Hold, or Width checks are in place; only functional (as

opposed to functional and timing) information is tested.  For this

specific information please refer to the SBus Specification document.

6.  Functional Blocks ("How it works")

6.1.  Monitor Module

The SBus Monitor code is primarily embedded in a single Verilog task

that is executed at the rising edge of SB_CLK.  Two state machines

(implemented as "case" statements) and the vast majority of error

checking appear in this task.  Illegal rising SB_BR_ lines are tested

asynchronously - they will generate an error immediately at the rising

edge of SB_BR_ if appropriate.

The primary task is gated by the SB_RESET_ signal.  When SB_RESET_ is

active (driven low) all internal state is reset, and a reset-timeout

counter watches SB_CLK.  No other checks occur until SB_RESET_ is

sampled inactive (high) at a rising edge of SB_CLK.

To make debugging easier, the boolean variable `debug' is used to gate

the $display of informative messages (internal state information, values

of significant variables, etc...).  This variable is preset to 0

(inactive), but can be forced active from interactive Verilog if the

Monitor is suspected of erroneous behavior after simulation time 0.

Furthermore, a task `display_gr_regs' can be called interactively to aid

debugging - it will fill the $gr_regs window (or create one if

necessary) with information about SBUSM internal variables.

There are two classes of errors that occur in the monitor: those that

$stop simulation; those that $display an error message, but do not stop

simulation.  Most printed messages are preceeded with "SBus Error: ",

but some with "SBus Warning" (it is NOT true that all "SBus Error"

messages stop simulation), and most are succeeded by simulation time

[$stime]).  Some messages which do not normally stop simulation will

stop simulation if the variable `debug' is set true (1).

A separate task called `display_version;' is called once when the

monitor initializes itself.  It can also be called interactively.

To use the monitor in batch mode, output can be redirected to a log_file

of your choice with the task: `output-file("file-name");' The log_file

defaults to "stdout" (the screen) at simulation time zero, so the task

must be called after that time to be effective.  Supplying the argument

"stdout" will do the right thing.  Simulations are $stop'd ONLY if the

log_file is "stdout".  The "file-name" argument is currently limited to

16 characters (including ".ext").

Please note that since `output_file' and `debug' are initialized at

simulation time 0, other settings of those options attempted at

simulation time 0 will be overridden.

6.1.1.  State Diagrams

These diagrams are not yet available in ASCII format.

6.1.1.1.  Transfer Initialization State Machine

6.1.1.2.  Acknowledge State Machine

6.1.2.  Protocol Monitor Messages

"SBus Monitor: Version %s", version

"SBus Monitor: Log File Name %s could not be opened", file_name

"SBus Error: Illegal br_ deactivation at %0d",$stime

"SBus Error: SEL_ pins illegal at %0d",$stime

"SBus Error: Device accessed improperly after rerun at %0d",$stime

"SBus Error: Ack code is reserved and illegal - at %0d",$stime

"SBus Error: Ack Timeout at %0d",$stime

"SBus Error: Ack pins must be driven at %0d",$stime

"SBus Error: Illegal transfer size at %0d",$stime

"SBus Error: Too many dynamically sized transfer ack's at %0d",$stime

"SBus Error: Improper address for nth dynamically sized transfer at %0d",$stime

"SBus Error: Ack pins not driven HIGH at %0d",$stime

"SBus Error: Too many burst acks at %0d",$stime

"SBus Error: Ack pins not released at %0d",$stime

"SBus Error: Ack pins released in middle of Burst Cycle at  %0d",$stime

"SBus Error: Ack pins are XXX at %0d",$stime

"SBus Error: sb_as_ deactivated before all required ack's occurred at %0d",$stime

"SBus Warning: sb_d pins undriven after ack at %0d",$stime

"SBus Error: No ReRun Ack allowed Mid-Burst. Time: %0d",$stime

"SBus Error: Illegal sb_siz at time %0d",$stime

"SBus Error: Illegal to change ack size during bus sizing.  Time %0d",$stime

"SBus Error: 32bit word transfer address must be aligned (00==LSB's) at %0d",$stime

"SBus Error: Late 32 bit ack at %0d",$stime

"SBus Error: No data present to read! Time: %0d",$stime

"SBus Error: 16bit transfer must be aligned.  Time: %0d",$stime

"SBus Error: Undetermined -  at %0d",$stime

"SBus Error: Transaction to or from wrong device during dyn.sized transfer at %0d",$stime

"SBus Error: sb_rd undriven during address strobe at %0d",$stime

"SBus Warning: This better be mux'd va on these data lines at %0d",$stime

"SBus Error: sel_ pins released during Slave Cycle at %0d",$stime

"SBus Error: sel_ not for same device as previous dyn.sized transfer at %0d",$stime

"SBus Error: siz or pa pins not driven during address strobe at %0d",$stime

"SBus Error: rd siz or virtual address (data) not driven during DMA at %0d",$stime

"SBus Error: rd not driven at data cycle of DMA at %0d",$stime

"SBus Error: Data lines driven during attempted read operation at %0d",$stime

"SBus Warning: Parity Error on data lines at - %0d",$stime

"SBus Error: bg_'s not driven properly!: sb_bg_[7:0] = %0b",sb_bg_

"SBus Error: sel_'s not driven to legal state: sb_sel_[7:0] = %0b",sb_sel_

"SBus Error: Short Reset Pulse - only %0d (<< 512 cycles)",reset_cycle

"SBus Error: SB_LateError_ pin driven before end of cycle at %0d",$stime

"SBus Error: Shouldn't be driving ack pins at %0d",$stime

"SBus Error: sb_as_ must always be driven by SB controller. Time: %0d",$stime

"SBus Error: rd, siz, and pa pins must be driven during as_ asserted. Time: %0d",$stime

"SBus Error: BG_ rose before appropriate end of cycle:"

"SBus Error: Monitor may be in incorrect internal state!"

"SBus Error: Ack_state is %0d, which means it's in outer space at %0d",ack_state,$stime

6.2.  Driver Routines

Each Driver routine is its own task, and each routine operates

independently from the others.  The SB_RESET_ pin is monitored by the

module, though, and disables any active routine when the SB_RESET_ pin

becomes active.  Remember that if a Driver task is executed before a

previous call to the same task has completed, the old call will have

it's variables smashed by the new call.

Many of the tasks require number_of_acks as a parameter.  The tasks

specifically do NOT decide what size or how many acks need to be driven

so that all permutations of Dynamic Bus Sizing may be tested.

Like the Monitor, there is a separate `display_version' task which is

executed when the code is initialized.  The task may be called

interactively at any time.

6.2.1.  User Definable Variables, Parameters, and Memory.

There are several user definable variables associated with the driver

routines, some of which are declared as registers so that they may be

easily changed during simulation runs, and some which are fixed at

compile time for the memory subsystem operation.

The register `precision' refers very specifically to precisely how many

rising edges of SB_CLK occur during the Translation Cycle of DMA (not

including the edge on which Virtual Address is detected n othe SB_D

lines).  Its value is preset to 0 at simulation time 0, thereby causing

the number of delay clocks to be random.  The register is constrained to

be 3 bits wide.  Remember that this variable cannot be set at simulation

time 0.

Two other registers are available to allow for more control of burst

cycle data: `dw_increment' and `dr_increment'.  When some ASIC has

mastership of the bus, and the "ack_data" task is being used to simulate

the slave device responding to the ASIC (implying that a Read Cycle is

occurring) the `dr_increment' resgister will determine how much will be

added to the data of ack_data's response during each subsequent burst

acknowledge.  Thus the data will always change by a well-defined (easy

to verify) amount.  The `dw_increment' register affects the data driven

onto the data bus by all "burst_write" routines.  Both registers are

preset to the value 1 at simulation time 0.

Three Parameters, `slot', `addr_start', and `addr_end', can be set with

"defparam's" in the level of verilog code in which SBUSD is

instantiated.  They control where a "virtual" memory subsystem (sbmem)

will be located.  The `slot' is preset to 0, and the address bounds are

initialized from 0 to 'h1fff (allocating a default 2Kx32bit Words) -

`addr_start' is the lower bound.  Instantiating SBUSD with the `slot'

parameter set to 7 will cause sbmem to be accessed whenever the 7th

SB_SEL_ line is active.

Attempts to read data from "memory" locations not previously written to

will generate 'hX on data lines (the SBUSM will halt simulation if this

happens).  Bytes and Half-Words are written into and read out of their

appropriate locations by all SBUSD tasks if the slot and address range

parameters are in bounds during a transaction.  The task `dump' is

provided to display the contents of the memory interactively.  This task

will $stop immediately after it is run, so it should be used exclusively

interactively.  The task `fill' can be used to preset sbmem contents.

(See section 6.2.2.7 for further info about memory related tasks).

One more parameter is available and is specifically for use when a Host

Controller model is present.  Since there is an SBus rule that SB_AS_

must be driven at all times, SBUSD is more than happy to do so.

Unfortunately, this constant drive will not be overdriven by The

Controller ASIC.  The parameter `drive_sb_as' is preset to 1 - this will

cause SBUSD to always drive SB_AS_.  When the Controller chip is present

during any simulation, it should be driving SB_AS_, and `drive_sb_as'

MUST be set to 0 using a defparam statement.

Note: Verilog parameters may also be set at instantiation of a module

via an alternate method: module_name #(par1, par2,...) instance

(pinlist...); The parameters for SBUSD are in the following order:

`drive_sb_as', `start_addr', `end_addr', `slot'.  The following line

will instantiate SBUSD with ONLY the drive_sb_as parameter (re)set to 0:

SBUSD #(0) sbusd (pinlist...);

6.2.2.  Driver Tasks

6.2.2.1.  init_bus

This task simply drives all SBus pins (except sb_reset_) to a known

state: SB_BR_, SB_BG_, SB_SEL_ are driven to 'hz and should be pulled up

externally; SB_AS_ driven high when not overridden by `drive_sb_as'; all

other signals are tristated.  This routine was necessary to present some

reasonable initial state to the Monitor during testing.  It might also

be useful to eliminate some useless initialization cycles for ASIC tests

- the task takes no arguments.

6.2.2.2.  ack and ack_data

Both of these routines simulate a slave device responding with acks to

some master.  `ack_data' drives the data lines with some (user supplied)

data word on the clock cycle after its ack, while `ack ' does not.

sbusd.ack (ack_code, number_of_acks, interval_between_acks,

random_intervals?, error_delay);

sbusd.ack_data (ack_code, number_of_acks, data_word,

interval_between_acks, random_interval?, error_delay);

`ack_code's:	0, 1 or 3	will drive one of the two reserved codes or

ERROR ack onto the SB_ACK_ lines.

2		will present a RERUN

7		will drive WAIT states (all 1's) onto the ack

lines, but this should never be necessary.

8, 16, or 32	will drive 8, 16, or 32 bit acks, respectively.

Please note that several other ack_codes will work, which remain for

debugging purposes.  I highly recommend that only codes 1, 2, 3, 8, 16,

or 32 be used.

`number_of_acks' is how many times you want an SB_ACK to be driven.

This is a 32 bit integer, so invalid numbers of acks are accepted

(though the Monitor will stop simulation if there are too many acks).

`interval_between_acks' determines the number of wait states inserted

between acks.  This value is limited to 3 bits.

`random_interval?' is a boolean variable which will make the

`interval_between_acks' random when set true.

`error_delay' is the number of valid acks to appear before an ERROR

(code 1 or 3 - the choice is random) is driven onto the SB_ACK_ lines.

This should be set to 0 (zero) to have no effect, and should not be set

greater than `number_of_acks'.  Please note that since no error ack is

given when the variable is set to 0, you must specify either a 1 or a 3

as the ack_code to cause either task to give an immediate error_ack.

`data_word' should be specified as 32'h and will be presented as the

first data word ONLY.  All data words for the rest of the burst will be

incremented by the value of `dr_increment'.  This argument will be

ignored if current SB_PA and SB_SEL_ indicate a read from the built-in

memory sub-system.

6.2.2.3.  read, burst_read, write, and burst_write

Each of these routines simulates an access by the System Master (SBus

Controller) to another device on the SBus.

sbusd.read (which_slave, port_siz, va);

sbusd.burst_read (which_slave, port_siz, va, number_of_acks);

sbusd.write (which_slave, port_siz, va, data_word);

sbusd.burst_write (which_slave, port_siz, va, data_word, number_of_acks);

`which_slave' is an integer from 0 to 7: 0-3 are the SBus Slots ; 4-6

are additional select lines.  SB_SEL_[`which_slave'] is the single SBus

select line driven low.

`port_siz' is interpreted directly based on the chart in the SBus

Specification:

0 ==>4 bytes			4 ==>16 bytes

1 ==>1 byte			5 ==>32 bytes

2 ==>2 bytes			6 ==>Reserved (Illegal)

3 ==>Reserved (Illegal)		7 ==>8 bytes

`va' is the address that gets driven onto the SB_PA lines.

The dma routines use this as virtual_address AND physical_address.

6.2.2.4.  dma_master

This task simulates the SBus Controller acting as a TRIVIAL CASE arbiter

for potential DMA masters.  It is NOT running continuously like a true

arbiter, rather it is to be called when an ASIC requires mastership of

the bus, and grants the bus until the completion of one slave cycle.  It

simply waits for SB_BR_, then immediately asserts the appropriate

SB_BG_.  Translation cycle delay (SB_CLK's before AS_ and PA are driven)

is controlled by the register `precision' (`precision' = 0 implies

random delay, and is the default - refer to section 6.2.1.).  Acks are

counted before SB_AS_, SB_PA, and SB_SEL_ are released.

If your ASIC needs to be a DMA Bus Master, this is the task to call:

sbusd.dma_master (which_master, which_slave, va, number_of_acks);

The only new parameter here is `which_master', whose behavior is very

much like `which_slave's except that a single SB_BG_ line is driven

based on the integer (0 - 7) supplied.

Also please note that if `va' is specified as "'hz", that the address

asserted on the SB_PA will be the same as the address that had appeared

on the SB_D line for translation.  Thus the designer has the option to

echo back the multiplexed address, or to ignore the multiplexed address

completely in favor of a non-`'hz' specified `va'.

6.2.2.5.  dma_slave_X

There are four dma_slave routines which are to be used by ASIC's that

will be dma_slaves.  To simplify stimulus files, these routines do not

require the dma_master routine.  These tasks act as both System Master

and Alternate Bus Master by granting themselves the bus (immediately) as

well as driving the SB_RD and SB_D pins (after `precision' translation

cycle delay).  To test a Controller, use the dma_alt_X routines

described below.

dma_slave_read (which_master, which_slave, atomic?, port_siz, va);

dma_slave_burst_read (which_master, which_slave, atomic?, port_siz,

va, number_of_acks);

dma_slave_write (which_master, which_slave, atomic?, port_siz, va,

data_word);

dma_slave_burst_write (which_master, which_slave, atomic?, port_siz,

va, data_word, number_of_acks);

The variable `atomic?' will determine the state of the chosen SB_BR_

line immediately following the assertion of SB_BG_: a 1 will hold the

SB_BR_ line active (indicating an atomic transaction), and a 0 will

release SB_BR_ normally.  Atomic transactions are not well dealt with by

these routines - Problems will be worked around as necessary.  If the

variable is set to "2", it is assumed that the ASIC itself will control

the SB_BR_ line.

6.2.2.6.  dma_alt_X

The four dma_alt routines are intended to test a Controller chip only.

The dma_slave routines function like the dma_alt+dma_master routines;

The alt routines don't include all of the same drivers as the slave

routines, so that the MSI is forced to drive those signals itself.  They

do take the exact same parameter lists as their dma_slave counterparts.

6.2.2.7.  dump and fill

Both of these tasks refer to the builtin memory construct.  They can be

used to preset and view the sbmem contents.  The `dump' task takes two

BYTE addresses, and displays all WORDS whose byte addresses are in

range.  The `fill' task forces the supplied BYTE into the given BYTE

address (so that the sbmem can be intialized as the user see's fit - the

sbmem construct is not otherwise intialized, so it's contents are all

X's).

dump (initial_byte_address, final_byte_address);

fill (byte_address, data_byte);

It may take a little longer to fill a large segment of memory by bytes

rather than by words, but this was the preferred method of the most

intense user of this software.

7.  Timing Diagrams

Please refer to the SBus Revision A Specification for relevant timing

diagrams.

8.  Revision History:

Rev 0.1: 2/27/90 Preliminary draft of this document.