Custom Neighborhoods Using the User Connector

The user connector, a pair of 50-pin male flat cable headers on the upper

right edge of the CAM board (see the Hardware Manual for further details, and

the file PINOUT.DOC) has pins carrying all the neighbor output signals and

table address input lines of CAM.  This makes it possible for the user, with

modest effort, to implement rules depending on other combinations of neighbors

than those allowed by the standard major neighborhoods N/VONN, N/MARG-HV,

N/MARG-PH, and N/MOORE; and standard minor neighborhoods &/PHASES, &/CENTERS,

and &/HV.  Using the user connector, any set of up to 12 neighbor and

pseudoneighbor signals may be used to determine the next state of CAM-A

(planes 0 and 1), while another arbitrary set of up to 12 signals determine

the next state of CAM-B (planes 2 and 3).

Physically, a custom neighborhood is most easily implemented by

custom-wiring a pair of female flat-cable headers with individual wires

unravelled from standard flat cable.  When these custom-wired female headers

are plugged onto user connector on the CAM board, the desired connections are

established.  The pair of custom-wired female headers can later be unplugged

from the CAM and set aside for subsequent experiments involving the same

custom neighborhood.

The user connector contains four kinds of signal sources that can be used as

neighbors or pseudoneighbors:

a) The 36 "plane outputs" comprising all Moore neighbors of the four bit

planes, named as indicated by the examples

N0  = north neighbor on plane 0,

C2  = center cell on plane 2, and

SW3 = southwest neighbor on plane 3.

etc.

b) Four "Phase bits" P0, P1, P2, and P3, which can be read by CAM as

pseudoneighbors PHASE, PHASE', &PHASE, and &PHASE' respectively, and whose

value can be altered between steps by the software using commands such as

1 IS <PHASE>    (sets P0 to 1)

0 IS <PHASE'>   (sets P1 to 0)

2 IS <PHASES>   (sets P1 to 1 and P0 to 0)

3 IS <&PHASES>  (sets P2 and P3 both to 1)

The important thing to remember about these phase bits is that their value

remains constant for all sites throughout a step.  Some or all of these phase

bits can be accessed through the standard neighborhoods, using the commands

N/MARG-PH and &/PHASES, but their presence on the user connector allows them

to be used in arbitrary combinations with other neighbor signals.

c) Two "parity bits" HT and VT which give the currently scanned site's

horizontal and vertical address, mod 2, relative to the chosen origin (the

parity of the origin can be changed between steps by commands such as 1 IS

<ORG-HV>, which sets the horizontal origin odd, vertical even).  Like the

phase bits, these parity bits are accessible from some of the standard

neighborhoods (e.g. &/HV), but are put on the user connector to make them

accessible in arbitrary combinations with other signals.

d) Four plane-edge signals H00, HFF, V00, and VFF, which indicate when the

currently scanned site is in the first or last column or row of the 256-by-256

array.  These signals are "active low" which means that they are 1 most of the

time but assume the value zero under the following conditions

V00 = 0  for sites in row 00 (top edge of screen)

VFF = 0  for sites in row FF (bottom edge of screen)

H00 = 0  for sites in column 00 (left edge of screen)

HFF = 0  for sites in column FF (right edge of screen)

These signals are not part of any of the standard neighborhoods.  They can be

used to provide boundary conditions at the edges (e.g. an absorbing boundary

for diffusing particles), or to modify the wraparound, as is done for example

in the experiments 64KWIDE.EXP and MORE64K.EXP, which use the edge signals to

define a spiral wraparound that turns the 256-by-256 system effectively into a

65536-by-1 site system useful for simulating very large one-dimensional

cellular automata.

Besides these OUTPUT pins, the user connector contains two kinds of

INPUT pins that are useful in realizing custom rules and neighborhoods.

The most important input pins on the user connector are the twenty "Table

Address Inputs" used as custom inputs to the tables.  Of these twenty, ten,

designated UA02 through UA11, go to the tables controlling CAM-A and the other

ten (UB02 through UB11) go to the tables controlling CAM-B.

When standard neighborhoods such as N/MOORE &/CENTERS are in effect the

signals presented to these pins are ignored by CAM, but if N/USER or &/USER is

in effect, they are routed as addresses to the tables.  For more details see

the User's Guide, which also gives details on the Forth word == used to assign

convenient names to the user neighbors.  Useful examples may be found in the

annotated experiment 64KWIDE.EXP and the more advanced MORE64K.EXP, as well

as in Chapter 10 of the User's Guide.

Besides the table address inputs, the user connector contains four "Plane

Input" pins UI0, UI1, UI2, and UI3, which bypass the internal tables entirely.

Normally signals presented at these pins are ignored, but appropriate commands

such as "EXTERN-A," defined on screen 1 of the file MORE64K.EXP, cause CAM-A,

or CAM-B (or both) to take the new state of the planes from the plane inputs

instead of from the internal tables.

These plane inputs are used in a fairly trivial way in MORE64K.EXP, to

implement some simple data transfers by simply wiring a plane output to a

plane input.  For example, wiring the plane output S3 to the plane input UI3

causes each site of plane 3 to take its next state from the present state of

its southern neighbor, instead of from the internal CAM-B tables, whenever

EXTERN-B is in effect.

A more sophisticated and powerful way of using the plane inputs would be

to use active external hardware (e.g.  gates or a clocked RAM look-up table)

to COMPUTE the value to be supplied to the plane input pins.  In this case the

external hardware would be substituting for the internal table, and if it were

complicated enough, could overcome some of the limitations of the internal

table such as the restriction to twelve neighbors.  Of course, in this case

the female headers plugged onto the user connector would not be functioning as

a simple plugboard, but rather as a link to active external hardware, (e.g.

another card) which would require its own power and ground.  The external

hardware would not, however, require its own clock, since the clock signal of

the CAM is already available on the user connector.

Another, less ambitious but probably equally useful, way of overcoming the

twelve-neighbor limitation would rely on the internal tables, with their limit

of twelve input lines, but would do a small amount of preprocessing of the

available neighbor and pseudoneighbor signals before feeding them in as table

address inputs.  For example, in the experiment 64KWIDE.EXP (which see) the

eastern spiral-wraparound neighbor is computed within the tables from the east

neighbor, northeast neighbor, and the signal HFF, thereby using three address

lines to compute one spiral neighbor.  If this computation were performed

externally, only the result would have to be fed in, saving two address lines.

The computation (a choice between E and NE under the control of HFF) could

be performed by very simple external hardware consisting of a single

data-selector chip such as 74F157 (in fact, one 74F157 chip could compute

spiral east neighbors on all four bit planes, and another could compute spiral

west neighbors on all bit planes).  The only caution in performing such

preprocessing is the need to avoid delaying the signals too much on their way

to being used as table address inputs.  This can be achieved by using only one

stage of preprocessing logic, using fast chips, and avoiding long wires

between the preprocessing logic and the user connector.  The pair of 74F157

chips would require their own ground and power (+5 volts, obtained from the PC

power supply or bus), and, for good design, their own bypass capacitors, but

otherwise would be scarcely more complicated or bulky than a passive

plug-board type customization of the user connector.